FACILITY LAYOUT

MATERIAL FLOW ANALYSIS REPORT

Comparative Evaluation of Master Flow Pattern A vs. Pattern B

| Document Type: | Technical Analysis Report – Industrial Engineering |
| --- | --- |
| Discipline: | Facility Planning & Material Handling Systems |
| Methodology: | Systematic Layout Planning (SLP) ¦ Quantitative Flow Analysis |
| Standards Reference: | OSHA 1910.176 ¦ MHI ASME MH1 ¦ AISI Layout Guidelines |
| Scope: | 8 Component Part Families ¦ 2 Layout Alternatives ¦ 10 Evaluation Criteria |
| Prepared By: | Industrial Engineering Department |
| Classification: | Internal Technical Document |

# PAGE 1: ANALYSIS OF MASTER FLOW PATTERN A

1.1  Pattern Overview & Flow Architecture

Master Flow Pattern A adopts a horizontal linear (L-to-R) flow architecture, in which 8 discrete component part families enter from the left via a common Receiving/Storage zone and progress sequentially through individual processing lanes toward the Assembly Line and Dispatch on the right. Each part family occupies a dedicated horizontal lane, and all lanes converge at shared Assembly stations (A1, A3, A4, A5, A7) before routing to the Quality Control Bay and Warehouse. The physical arrangement places Receiving → Storage → Fabrication → Assembly → QC → Dispatch along a single directional axis — a classical I-shaped flow, consistent with AISI layout planning principles for medium-complexity fabrication environments.

Component Part Families & Processing Steps

| Part Family | Steps | Process Sequence (Pattern A) | Assembly Pt. |
| --- | --- | --- | --- |
| 1. Base Plate | 6 | Measuring Bench → Waterjet Cutter → Vertical Milling → Deburring → Surface Grinder → Inspection | A1 |
| 5. Mounting Bracket | 6 | Measuring Bench → Band Saw → Vertical Milling → Deburring → Surface Grinder → Inspection | A1 |
| 6. Handle Bar | 7 | Measuring Bench → Cold Saw → Deburring → Bending → Inspection → Polishing → Inspection | A1 |
| 2. Nozzle Bar Frame | 6 | Measuring Bench → Cold Saw → Milling → Drill Press → Welding → Deburring → Inspection | A3 |
| 3. Elbow Pipe | 6 | Measuring Bench → Cold Saw → Deburring → CNC Bending → Milling → Belt Sander → Inspection | A4 |
| 4. Pipe Fitting 1 | 5 | Measuring Bench → Cold Saw → Deburring → Milling → Buffing → Inspection | A4 |
| 12. T-Pipe | 8 | Measuring Bench → Cold Saw → Milling → Drill Press → Deburring → Welding → Buffing → Inspection | A5 |
| 11. Pipe Fitting 2 | 6 | Measuring Bench → Cold Saw → Deburring → Milling → Buffing → Inspection | A7 |

1.2  Identified Strengths

1.3  Potential Bottlenecks & Flow Inefficiencies

Backtracking Risk (Critical Deficiency — Score: 5/10): This is Pattern A's most significant engineering weakness. Multiple part families share identical machines (Cold Saw, Milling Machine, Drill Press, Deburring Machine, Welding Station) but these machines appear at non-sequential positions across lanes. For example:

This represents a classic backtracking pattern wherein WIP must traverse already-visited floor zones, increasing travel distance by an estimated 18–24% over a clean sequential path. AISI Best Practice recommendation: shared equipment should be positioned at a common functional work center rather than duplicated across lanes.

Lateral Cross-Traffic at Assembly Nodes: Five distinct assembly nodes (A1, A3, A4, A5, A7) are distributed along the right-hand edge of the fabrication area. Material from 8 separate horizontal lanes must converge laterally onto the Assembly Line. Without clearly demarcated lateral transfer aisles, this creates fork-lift and personnel cross-traffic — a known contributor to workplace incidents under OSHA 29 CFR 1910.178.

Shared Equipment Utilisation Conflict: Measuring Benches, Cold Saws, and Inspection Tables appear in every lane but are shown as individual units. If these represent shared resources rather than dedicated per-lane equipment, Pattern A creates queue build-up at high-utilisation stations, particularly at Inspection Tables (present in all 8 lanes) and Cold Saws (6 of 8 lanes).

1.4  Material Handling Equipment Requirements

| MHE Category | Equipment Type | Application |
| --- | --- | --- |
| Primary Aisle Transport | Counterbalance Forklift (2.5T) | Primary horizontal transfer along main aisle (L→R) |
| Inter-Lane Transfer | Reach Truck or Pallet Jack | Lateral movement to Assembly Line nodes |
| WIP Between Stations | Gravity Roller Conveyor | Within-lane sequential station-to-station transfer |
| Receiving & Dispatch | Dock Leveller + Electric Pallet Truck | Inbound/outbound at facility extremities |
| Inspection Transfer | Scissor Lift Table | Part presentation at Inspection Tables for QC |
| Heavy Blank Handling | Overhead Crane (1T jib) | Cold Saw & Waterjet Cutter loading (Base Plate) |

1.5  Quantitative Flow Efficiency Metrics (Pattern A)

Note: Metrics below are engineering estimates based on assumed 12 m bay depth, 8 m lane pitch, and 2 m aisle widths. Actual values require facility dimension confirmation.

| Metric | Value | Notes |
| --- | --- | --- |
| Total estimated travel distance | ~1,850 m / shift | Based on 8 part families × avg. 231 m/part cycle |
| Average inter-station distance | ~3.2 m | Stations within-lane; longer for cross-lane moves |
| Backtrack incidence rate | ~22% of total moves | T-Pipe & Nozzle Bar Frame primary contributors |
| Assembly convergence congestion | HIGH | 5 assembly nodes, 8 lanes converging laterally |
| Estimated floor area utilisation | ~68% | Wide aisles required for lateral fork travel |
| Evaluation weighted score | 614 / 810 | Refer to Evaluation Sheet, Design 1 column |

| PATTERN A SUMMARY | Horizontal linear flow. Strong R&S separation and flexibility. Critical weakness: backtracking risk (5/10) and lateral assembly convergence congestion. Estimated travel: ~1,850 m/shift. Weighted evaluation score: 614/810. |
| --- | --- |

# PAGE 2: ANALYSIS OF MASTER FLOW PATTERN B

2.1  Pattern Overview & Flow Architecture

Master Flow Pattern B restructures the facility into a vertical columnar (top-to-bottom) flow architecture. Each of the 8 component part families is allocated a dedicated vertical processing column, with material entering at the top (Receiving Line) and progressing downward through all fabrication stations before joining the shared Assembly Line at the bottom. This configuration is architecturally equivalent to a U-shaped or parallel-lane flow when viewed in section, and aligns strongly with cellular manufacturing principles — each column functions as a quasi-independent manufacturing cell. The Receiving → Storage → Fabrication → Assembly → QC → Warehouse path is maintained, but the dominant flow vector is vertical (top → bottom) rather than horizontal.

2.2  Identified Strengths

2.3  Potential Bottlenecks & Limitations

Reduced Layout Flexibility (Score: 6/10): Pattern B's principal weakness. Adding a new processing step within a column requires vertical elongation of that column, which may conflict with building depth constraints. In contrast to Pattern A's horizontal extensibility, Pattern B is constrained in the depth dimension. Cross-family machine sharing is also more difficult to implement without introducing inter-column travel.

Receiving Area Density: All 8 part families share a common Receiving Line at the top of the facility. During peak inbound delivery periods, this creates a staging bottleneck as 8 column queues compete for the same receiving dock. A dedicated sequenced scheduling system (e.g., kanban pull signals per column) is recommended to mitigate this risk.

Shipping/Dispatch Path Clarity: Unlike Pattern A where Dispatch is clearly positioned at the right-hand facility extreme, Pattern B routes completed assemblies from the bottom Assembly Line to the Quality Control Bay and then to the Warehouse in a shared pathway. The Shipping zone is less spatially isolated than in Pattern A, increasing the risk of outbound/in-process material intermixing without clear floor marking and traffic management protocols.

2.4  Comparative Analysis: Pattern A vs. Pattern B

| Evaluation Criterion (Wt.) | Pattern A Score | Pattern B Score | Preferred Pattern |
| --- | --- | --- | --- |
| Optimum Material Flow | 7 | 8 | Pattern B |
| Continuous Flow | 6 | 9 | Pattern B |
| Receiving & Shipping Location | 8 | 7 | Pattern A |
| Optimum Personnel Flow | 6 | 8 | Pattern B |
| Health & Safety Compliance | 7 | 8 | Pattern B |
| QC Aspects Attainability | 7 | 8 | Pattern B |
| Layout Flexibility | 8 | 6 | Pattern A |
| Minimum Backtracking | 5 | 9 | Pattern B |
| Building Compatibility | 7 | 8 | Pattern B |
| Minimum Distance Travelled | 6 | 8 | Pattern B |
| WEIGHTED TOTAL (out of 810) | 614 | 792 | ✓ Pattern B |

2.5  Material Handling Equipment Requirements (Pattern B)

| MHE Category | Equipment Type | Application |
| --- | --- | --- |
| Column Internal Transfer | Gravity Roller / Flat Belt Conveyor | Top-to-bottom within-column WIP movement |
| Inter-Column Buffer | Manual Tugger + Flat Trolley | Moving parts between columns at Receiving only |
| Assembly Feed | Floor-level Powered Conveyor | Each column base to its Assembly node |
| Receiving Dock | Dock Leveller + Multi-lane Pallet Flow Rack | Sequenced inbound per column |
| QC Bay Transfer | Scissor Lift Table | Finished assembly presentation for final inspection |
| Warehouse Dispatch | Reach Truck + Pallet Wrapper | Outbound from QC Bay to Warehouse/Shipping |
| Overhead Handling | Overhead Bridge Crane (2T, bay-mounted) | Heavy part families (Base Plate, Handle Bar) |

2.6  Final Engineering Recommendation

RECOMMENDATION: Implement Master Flow Pattern B (Design 2)

Based on the Systematic Layout Planning (SLP) evaluation, Pattern B achieves a weighted score of 792/810 versus Pattern A's 614/810 — a 28.8% improvement in overall layout efficiency. Pattern B is the superior configuration on 8 of 10 evaluation criteria, with particular dominance on the highest-weighted criteria: Minimum Backtracking (Wt. 10), Continuous Flow (Wt. 9), Optimum Personnel Flow (Wt. 10), and Building Compatibility (Wt. 9). The estimated ~33% reduction in total travel distance (~610 m/shift) directly translates to reduced material handling costs, improved throughput, and lower operator fatigue — all primary KPIs in a lean fabrication environment.

The only criteria where Pattern A outperforms are Receiving/Shipping Location (+1 point) and Layout Flexibility (+2 points). These gaps can be mitigated in Pattern B through: (a) clear floor marking and traffic separation at the Receiving Line, (b) colour-coded column boundaries to maintain visual management standards, and (c) reserved expansion zones at the base of each column to accommodate future process step additions.

| IMPLEMENTATION NOTES | 1. Apply 5S visual management (floor tape, shadow boards) to delineate column boundaries. 2. Install kanban pull signals at column entry (Receiving Line) to prevent staging congestion. 3. Conduct time-study on shared Assembly nodes post-implementation to confirm takt-time alignment. 4. Review column depth against building structural grid before finalising bay assignments. 5. Maintain minimum 3.0m primary aisle width per OSHA 1910.176(a) for forklift routing between columns. |
| --- | --- |

| PATTERN B SUMMARY | Vertical columnar flow. Eliminates backtracking, achieves continuous top-down flow, reduces travel ~33%. Weighted evaluation score: 792/810. RECOMMENDED for implementation over Pattern A. |
| --- | --- |